Homework 3

Department of Mechanical and Aerospace Engineering

University of California, San Diego

MAE 150

Lawrence Custodio

A08739279

31 January 2013

Problem 2:

(a)

Nominal value:

1.1796e+04

Worst Case Error:

301.4720

Statistical Error:

243.2613

Calculations were done through MATLAB as follows: results show that worse case error > statistical error

%Defining the variables:

rho = 8960; drho = 10; %density + uncertainty

D = 2e-6; dD = 0.01e-6; %diameter + uncertainty

L = 3e-4; dL = 0.03e-4; %length + uncertainty

E = 128.8e9; dE = 0; %Modulus + uncertainty

% Function + Partial derivativesnd

f = (0.56/L^2)*sqrt((E*D^2)/(16*rho));

frho = -(7*D^2*E)/(400*L^2*rho^2*((D^2*E)/(16*rho))^(1/2)); %df/drho

fD = (7*D*E)/(200*L^2*rho*((D^2*E)/(16*rho))^(1/2)); %df/dD

fL = -(28*((D^2*E)/(16*rho))^(1/2))/(25*L^3); %df/dL

fE = (7*D^2)/(400*L^2*rho*((D^2*E)/(16*rho))^(1/2)); %df/dE

disp('Nominal value:');

disp(f);

%Worst Case Error

fwce = abs(frho)*drho + abs(fD)*dD + abs(fL)*dL + abs(fE)*dE;

disp('Worst Case Error:')

disp(fwce)

%Statistical Error

fstat = sqrt(frho^2*drho^2 + fD^2*dD^2 + fL^2*dL^2 + fE^2*dE^2);

disp('Statistical Error:')

disp(fstat)

(b) Uniform Distribution

Normal Distribution

(c) Results suggests that the distribution of the independent variables (L, E, ρ) will reflect on the distribution of the dependent variable (frequency). Uniform distribution for the dependent variable however, shows that there is a larger proportion of values along the mean versus the proportion of values at deviations ± from the nominal value.

Problem 3: CAM Design

(a)

(b) Plot Pressure Angles

Table for pressure angle values:

| CAM angle [θ] | 0 | 30 | 60 | 90 | 120 | 150 | 180 | 210 | 240 | 270 | 300 | 330 | 360 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pressure Angle [θ] | 0 | 50.812 | 55.631 | 41.781 | 8.394 | 0 | 0 | -20.337 | -25.658 | 0.055 | 0 | 0 | 0 |

The range of pressure angle values suggest that the harmonic rise may not be designed optimally as the pressure angle reaches >30o, while the fall and the dwell ranges are consistent of the desired range of pressure angle value.

Problem 4: SolidWorks

Lesson 5: Assembly

Wheelbarrow.sldasm

(Collapsed View)

(Exploded View)

Lesson 6: Finite Element Analysis

Displacement Figure

Stress figure of vonMises stress

Stress Iso-Clipping

Appendix: MATLAB Code

Problem 2:

clear;clc;

%Part A

%Defining the variables:

rho = 8960; drho = 10; %density + uncertainty

D = 2e-6; dD = 0.01e-6; %diameter + uncertainty

L = 3e-4; dL = 0.03e-4; %length + uncertainty

E = 128.8e9; dE = 0; %Modulus + uncertainty

% Function + Partial derivativesnd

f = (0.56/L^2)*sqrt((E*D^2)/(16*rho));

frho = -(7*D^2*E)/(400*L^2*rho^2*((D^2*E)/(16*rho))^(1/2)); %df/drho

fD = (7*D*E)/(200*L^2*rho*((D^2*E)/(16*rho))^(1/2)); %df/dD

fL = -(28*((D^2*E)/(16*rho))^(1/2))/(25*L^3); %df/dL

fE = (7*D^2)/(400*L^2*rho*((D^2*E)/(16*rho))^(1/2)); %df/dE

disp('Nominal value:');

disp(f);

%Worst Case Error

fwce = abs(frho)*drho + abs(fD)*dD + abs(fL)*dL + abs(fE)*dE;

disp('Worst Case Error:')

disp(fwce)

%Statistical Error

fstat = sqrt(frho^2*drho^2 + fD^2*dD^2 + fL^2*dL^2 + fE^2*dE^2);

disp('Statistical Error:')

disp(fstat)

%Part B:

%Uniform Distribution + Plotting

n = 1e4 ; %r sample-size

figure;

%Independent Variable Distribution: diameter

r2 = rand(n,1);

ud = 2*dD*r2+(D-dD);

subplot(222)

hist(ud,50);

xlabel('[m]')

title('Diameter')

%Independent Variable Distribution: length

r3 = rand(n,1);

ul = 2*dL*r3+(L-dL);

subplot(223)

hist(ul,50);

title('Length')

xlabel('[m]')

%Independent Variable Distribution: density

r4 = rand(n,1);

urho = 2*drho*r4+(rho-drho);

subplot(224)

hist(urho,50);

title('Density')

xlabel('[kg/m^3]')

%Dependent Variable Distribution: frequency

ufreq = (0.56*ul.^-2).*sqrt((E*ud.^2)./(16*urho));

subplot(221)

hist(ufreq,50);

title('Frequency')

xlabel('[Hz]')

%Normal distribution

figure;

%Diameter

r2n = sqrt(2)*erfinv(2*r2-1); %Normally distributed random number array

nD = D + (dD/3)*r2n; %Normal dist. formula

subplot(222)

hist(nD,50);

title('Diameter')

xlabel('[m]')

%Length

r3n = sqrt(2)*erfinv(2*r3-1); %Normally distributed random number array

nL = L + (dL/3)*r3n; %Normal dist. formula

subplot(223)

hist(nL,50);

title('Length')

xlabel('[m]')

%Density

r4n = sqrt(2)*erfinv(2*r4-1); %Normally distributed random number array

nrho = rho + (drho/3)*r4n; %Normal dist. formula

subplot(224)

hist(nrho,50);

title('Density')

xlabel('[kg/m^3]')

%Dependent variable: Frequency

nfreq = (0.56*nL.^-2).*sqrt((E*nD.^2)./(16*nrho));

subplot(221)

hist(nfreq,50);

title('Frequency')

xlabel('[Hz]')

Problem 3:

%CAM Design

clc; clear; close all;

L =20; %Given length of rise +fall

w = 600; %rotational speed

%Harmonic Rise

t1 = (0:120)*pi/180; %Range of Harmonic Rise (rad)

b1 = t1(end)-t1(1); %Beta value

y1 = (L/2)*(1-cos((pi*t1)/b1)); %Displacement

y1dt = (0.5*pi*L*w*b1)*sin((pi*t1)/b1);%Velocity

y1dt2 = (L/2)*((pi*w/b1)^2)*cos(pi*t1/b1); %Acceleration

%Dwell

t2 = (120:180)*pi/180; %Range of Dwell (rad)

y2 = 20*ones(1,length(t2)); %Horizontal line describing dwell

y2dt = zeros(1,length(t2)); %Zero velocity for dwell

y2dt2 = zeros(1,length(t2));

%345 Poly Fall

t3 = (0:90)*pi/180; %Range of fall (rad)

b3 = t3(end)-t3(1); %Beta for poly 345 fall

y3 = L-L*(10*(t3/b3).^3 - 15*(t3/b3).^4 + 6*(t3/b3).^5); %Displacement

y3dt = L-L*((30*w/b3^3)*t3.^2 - (60*w/b3^4)*t3.^3  + (30*w/b3^5)*t3.^4 ); %Velocity

y3dt2 = L-L*((60*w^2/b3^3)*t3 - (180*w^2/b3^4)*t3.^2  + (120*w^2/b3^5)*t3.^3); %Acceleration

%Dwell

t4 = (270:360)*pi/180;

y4 = zeros(1,length(t4));

y4dt = zeros(1,length(t4)); %Zero velocity for dwell

y4dt2 = zeros(1,length(t4));

%Graphing:

%Respective ranges for angles are converted in degrees:

tt = 0:120; %Harmonic Rise

tt2 = 120:180; %Dwell

tt3 = 180:270; %345 Poly Fall

tt4 = 270:360; %Dwell

t = [tt tt2 tt3 tt4]; %Angles

disp = [y1 y2 y3 y4]; %Displacement values

vel = [y1dt y2dt y3dt y4dt]; %Velocity

acc = [y1dt2 y2dt2 y3dt2 y4dt2]; %Acceleration

subplot(311)

plot(t,disp)

title('Displacement Profile ')

ylabel('Displacement[mm]')

xlim([0 360]);

ylim([0 30]);

subplot(312)

plot(t,vel)

title('Velocity Profile')

ylabel('Velocity [mm/s]')

xlim([0 360]);

subplot(313)

plot(t,acc)

title('Acceleration Profile')

xlabel('Cam angle [\theta]')

ylabel('Acceleration[mm/s^2]')

xlim([0 360]);

%Part B: Pressure angle

phi = atan((vel./w)./(35+disp)); %Pressure angle formula

phid = phi*180/pi; %Pressure angle values to degrees

figure

plot(t,phid)

title('CAM Pressure angle')

xlabel('CAM Angle [\theta]')

ylabel('Pressure Angle \phi[deg]')

%Table values for every 30 deg:

x = [t(1) t(31) t(61) t(91) t(121) t(152) t(182) t(213) t(243) t(273) t(304) t(334) t(364)]; % Selected angle values

pa = [phid(1) phid(31) phid(61) phid(91) phid(121) phid(152) phid(182) phid(213) phid(243) phid(273) phid(304) phid(334) phid(364)]; %Pressure angle values

table=[x;pa]

I verify that the above homework was generated by me  alone, without the help of any other person

Signature / Date